Mars technology product parameter sheet

DK-XVR7004GH  4CH 5M-N DVR

TVI/CVI/AHD/CVBS/IP 6 in 1

Key Feature：

Support H.265 encoding, ultra-low bitrates, compatible with H.264 surroundings;
Support VGA, HDMI full HD output simultaneously;

Support ONVIF; access third party;

Support multiple cloud technology, network penetration, alarm information pushed to your phone and other functions;

Support web, VMS, center platform management software MYEYE, provide SDK development;

Support various mobile monitoring (iPhone, Android);

Remote Access, With DNS (ARSP),easy to monitor remotely

Support UTC, control signal& video stream transmit through one single coaxial

Support XVI/AHD/TVI/CVI/CVBS/IP six signals;

Support intelligent analysis: Human detection, face detection;

Picture:

[image: image1.jpg]



Specification：
	
	Model
	DK-XVR7004GH

	System
	Main processor
	NT98321

	Interface
	Operating interface
	support 16-bit true color graphical menu interface and mouse operation

	
	Preview
	1/4

	Video
	Codec
	H.265

	
	Monitor quality
	VGA(1080P at most);HDMI(1080P at most)

	
	Preview ability
	4*5M-N@20fps（6 in 1）/4*4M-N@30fps（6 in 1）/4*1080P（6 in 1）default:5M-N

	
	Encode ability
	4*5M-N@20fps（6 in 1）/4*4M-N@30fps（6 in 1）/4*1080P（6 in 1）default:5M-N

	
	Decode ability
	4*5M-N@20fps（6 in 1）/4*4M-N@30fps（6 in 1）/4*1080P（6 in 1）default:5M-N

	
	Multi-mode input
	Analog：4*5M-N

hybrid：4*5M-N（analog）+2*1080P（network）

network：16*1080P;8*5M

	
	Motion Detection
	Each screen can be set to 192 (16 * 12) detection areas; multi-level sensitivity can be set (Limited local channel)

	Audio
	Audio compression
	G.711A

	
	Intercom
	support

	Record & playback
	Record mode
	manual>alarm>motion detection>timing

	
	Local Playback
	2ch(analog input mode)

	
	Search mode
	Search by time, calendar, event, channel

	Storage & backup
	Video storage
	HDD, network

	
	Backup mode
	Network, USB hard disk, USB burner, SATA burner

	External Interface
	Video input
	4ch BNC

	
	Video output
	1ch VGA,1ch HDMI

	
	Loop Vout
	N/A

	
	Audio input
	1ch 

	
	Audio output
	1ch 

	
	Alarm input
	0

	
	Alarm output
	0

	
	Network interface
	RJ45 10M/100M 

	
	PTZ control
	1*RS485, Support kinds of PTZ protocols

	
	USB
	2* USB 2.0 ports

	
	HDD
	1*SATA ports (Max6TB/per)

	General
	Power supply
	12V/2A

	
	Power Consumption
	<15W (without HDD)

	
	Working environment
	Temperature:0℃—+55℃，Humidity:10%—90%，Atm:86kpa—106kpa



