HD Pipe Inspection System

[image: image10.png]



[image: image1.png]


Manual
Please read this instruction manual carefully before using and keep it for future reference.
Attention

1. Please read the instruction carefully before using the product.
2. Do not drop or press hard.

3. If the user disassembles the machine by himself or damaged, the warranty is invalid.

4. The operation of the product should be standardized, otherwise it will cause camera  

defect or cable damage.

5. If the product is upgraded, the instruction manual may be different from the product,  

will be no further notice.

Product overview

1. The camera diameter is 25MM, which is suitable for diameter 27-125MM pipes.

2. Take original imported true color LCD monitor with ideal effects when viewed from 

different angles.

3. Use ultra-high-definition photosensitive chip and high-definition lenses, the  
definition is high.

4. Good quality night vision LED .

5. The night vision light can be adjusted according to the needs.

6. With sunshade.

7. High quality glass fiber detection cable.

8. With a large-capacity rechargeable lithium battery, it can show the battery power  
consumption state.
9. Stainless steel housing camera, durable.

10. Where to us this system: Water supply pipes, air conditioning pipes, cable pipes, pipe   
vacuum systems, lead  industry (PLUNBIN) pipelines, buildings underground pipelines, rescue work in small spaces, exploration of underground caves in the wild, scientific research on underwater riverbeds, etc.

Specification
	Display

	Display Size
	7” 

	Display Ratio
	16:9

	Resolution
	1024×768      

	Language
	Chinese, English

	Outdoor shading function
	With sun shield

	Control box and power input

	Support playback format
	Video

	
	Photo

	Storage medium
	Support SD card (Max: 64G)

	Battery amount display
	Battery amount can be viewed at any time on battery control board

	External power supply
	DC12V (Input voltage AC 100V~240V, Output DC:12.6V/1A)

	Lithium battery 
	12V 4400mA/H

	Whole system power consumption
	8W（Max）

	Lithium battery charging time
	6 hours

	Lithium battery work time
	6 hours

	Camera

	Sensor
	HD AHD SENSOR

	Light
	12pcs LED white light 　

	Size
	Diameter 25MM, length 110MM

	Package
	

	Aluminum box
	390*320*150mm


Product Introduction
1. Monitor (7" or 9")
A. Button function

1. REC/-: record, play, pause/add parameter
2. PLAY/+: video, photo, playback switch/exit

3. MENU/EXIT: Menu

4. > : previous video, photo/zoom out

5. <: next/next video, photo/zoom

6. POWER : display on/off
[image: image9.jpg]123456 TF




       7 inch                        9 inch
B. Button operation

1. Start

After connecting well, keep pressing POWER button for 3 seconds, the system starts to work.

2. Menu Settings

Press the MENU button to enter the menu page, and press the REC/-, > or < button to set various menu settings.

      1>Display brightness, contrast and chrome setting and reset

      2> Chinese, English and image flip settings

      3>Time setting

      4>Photo viewing, taking photos and memory card formatting operations

5>Video playback, video and memory card formatting operations, video 

audio switch

3. Recording operation

When insert a new memory card, please format the card firstly, press the REC/- button to start recording, the upper left corner of the screen will flash REC, and then press the REC/- button during the recording to exit the recording.

4. Take photo (REC/-)
Press the PLAY/+ button, the camera icon will appear in the upper left corner, and press the REC/- button each time to take a picture.

5. Photo and video playback

Press and hold the PLAY/+ button to enter playback, press the > or < button, select the video or photo to view, and press the REC/- button to play the video, press again to pause playback.

6. Image scaling

In the monitoring or recording state, press > or < to reduce or enlarge the image.

7. Exit operation

In the menu or playback state, long press the PLAY/+ button to exit the current operation.

C. Remove SD card

1. Press the buttons on both sides of the monitor with both hands at the 
same time to pop up the display screen.

[image: image2.jpg]



2. On the left side of the monitor, press down SD card slowly to eject the 

SD card.

  [image: image3.jpg]



2. Camera and guide

1. Connector     2. Spring        3. Night vision light     4. Lens 
  5. 45mm guide   6. 125mm guide

   [image: image4.png]



3. Inside box introduction

A. Brake twist

  lock the cable by clockwise direction, release the cable by counterclockwise direction.

Note: 

1> Do not take the bake out, just slightly adjust.

2> Please lock the cable by the brake during the delivery.

3> Please release the brake if you use the system.

B. Cable wire loop

The cable must come from the cable wire loop.
  [image: image5.png]Cable Wire Loop  Brake Twist




C. Retractable cable

  Release the brake firstly, pull to release cable, push to retract the cable,
    [image: image6.png]> Push to retract cable

(=] ®

© @
|

N

Pull to release cable —=




D. Accessories area

    Open the cover, will see the accessories

    [image: image7.png]



E. Function    

    [image: image8.png]g
4

Indicator

2 HOLVOIONI A¥3LIVE °
°




1. POWER: Long press for 3 seconds to turn on the system; long press for  

3 seconds to turn off. 

2. Turn on the system, the battery indicator will light, totally 5 grade.

3. Light brightness adjustment: long press LED+ to brighter the camera

LED, long press LED- to week the brightness, or short press to adjust.

4. Charging Port: Use the standard charger to charge the system if the battery is lower.
Charging

1. Connect the charger output to the charging port.
2. Plug the charger input into the AC socket of AC100V-240V.
3. Red light means charging, green light means charged fully.

Note: 1. Do not use battery power to 0, need charge the battery timely if battery poor. If it is used excessively, the battery may be damaged and cannot be used again.

2. Do not use during charging, and do not overcharge.

Package
1. Aluminum box (with cable and EVA cotton, cable 20, 30, 40m optional)

2. Camera

3. HD monitor
4. 45MM guide

5. 125MM guide

6. 8G memory SD card

7. Charger
PAGE  

